# EE-492-S02-SpTp-Advance Digital Hardware-SP2017

Project #2 - Report

# Design and Verification of a 
SPI to JTAG Interface Adapter

Written by:  	Jordan D. Ulmer

Written by:  	Nathan Genetzky

Instructor:	Dr. Robert Fourney

| Design Initiated: | 2017-03-25 |
| --- | --- |
| Date Demonstrated: | 2017-05-03 |
| Report Submission: | 2017-05-05 |
| Git Repository: | https://bitbucket.org/adhd_digital_design/ee492 |

## Goal and Motivation:

Almost all hardware devices have JTAG connections which can provide data from registers or provide low level control of pins via a boundary scan chain. Unfortunately, most microcontrollers do not have libraries  or drivers for communicating using the JTAG protocol.  The goal of this project is to provide a way to allow users to use existing libraries to access data that is typically accessed via JTAG protocol.

## Table of Contents:

EE-492-S02-SpTp-Advance Digital Hardware-SP2017	1

Goal and Motivation:	2

Table of Contents:	2

Background:	3

System Components:	5

Hardware Design Components:	6

top_spi2jtag & top_hipe_spi	6

nios2_spi_pio	6

hipe_spi	8

hipe_jtag	9

spi2jtag	10

HDL Files	11

HDL Components for Top:	11

Software Design Components:	12

Spi2JtagApp	12

Software Files	12

Verification:	14

Verified jtag_tap_fsm via dev. board	14

Verified TAP state transitions	14

Verified IR and DR shifts	15

Demo Recap:	16

Future Work:	16

Appendix:	17

## Background:

Four connections are required for JTAG and multiple devices can be supported by daisy chaining the Test Data In (TDI) & Test Data Out (TDO) connections between the devices, Fig. 1. Serial Peripheral Interface (SPI) is another serial protocol; one that is more likely to have a library provided for a particular micro controller. The similarities between SPI are readily apparent by looking at SPI interfacing with multiple devices in a daisy chain configuration, Fig 2. Note that both protocols have bidirectional flow by providing two data pins; the  TDI of JTAG slave = Master Out Slave In (MOSI) of SPI master and TDI of JTAG slave = Master In Slave Out (MISO) of SPI master. The fourth signal is where these two protocols differ greatly. The JTAG Test Mode Select (TMS) signal requires more logic than Slave Select (SS), as seen in the timing diagram for a typical JTAG communication, Fig. 3. The TMS and TCK are used to control the TAP state machine, Fig. 4.

Figure 1: Multiple devices connected to a JTAG connection.

Figure 2: Multiple SPI slaves in daisy chain configuration.

Figure 3: Timing diagram for a typical JTAG communication.

Figure: 4:  JTAG finite state machine (IEEE 1149.1 JTAG)

## System Components:

Both software and hardware components were utilized to demonstrate the capability of the spi2jtag HDL module.  A general overview of one particular application of the SPI to JTAG (spi2jtag) hdl module is shown below(See Fig. 6).  Note that in the particular application demonstrated the “JTAG TDRs” is also a hdl module.   A more general use case could use JTAG signals, provided by spi2jtag, and be connected to a JTAG device that is external to the FPGA (See Fig. 5).

SPI Master

SPI

spi2jtag

JTAG (TMS)

JTAG Tap Controller

JTAG TDRs

JTAG (TDI/TDO)

IR &

STATE

Figure 5: Block diagram for SPI Master driving a JTAG Device (Tap Controller and TDRs) through spi2jtag.

FPGA

nios2_pio_spi

User PC

hipe_spi

altera_avalon_spi 
(3-wire Serial)

SPI

PIO

hipe_jtag

jtag_tap_ctrl

altera_avalon_pio

altera_nios2_gen2

(softcore processor)

jtag_fsm

spi2jtag

UART

nios2_terminal

altera_avalon_jtag_uart

altera_avalon_onchip_memory

(80kB - Software)

spi2jtag library

spi2jtag tests

Nios2 Instructions

COMM-Key

Required

Verification

JTAG

IR & STATE

Altera IP

Senior Design

EE-492

Figure 6: System Block Diagram for testing spi2jtag with a Nios2 processor as the SPI master.

## Hardware Design Components:

The important hardware modules are described in detail, and then a summary of each hdl file is provided.  For the convenience of the reader components designed in EE-492 by Jordan and Nathan are listed in black bold italics (ie. top_spi2jtag).  Individual Qsys components directly used from altera are listed in italics (ie. altera_avalon_spi).  Components used from the Hardware Integrated Prototyping Environment (HIPE) Senior Design project, of which both Jordan and Nathan were team members are listed in bold red (ie. Senior Design) (See key in Fig. 6)

### top_spi2jtag & top_hipe_spi

Several hdl modules were used to demonstrate SPI to JTAG communication; top_hipe_spi is equivalent in core functionality to top_spi2jtag, Fig.7. The nios2_spi_pio Qsys module uses Altera embedded IP. The SPI core in this Qsys module communicates with two SPI slaves in hipe_spi via spi_inf.

Figure 7: RTL for top_spi2jtag.

### nios2_spi_pio

Altera Qsys components, Fig. 8, were combined into a single Qsys module nios2_spi_pio.  The SPI Qsys component was configured as a SPI master; the timing configuration for the SPI Qsys component, Fig. 9, was determined by considering SPI-Clock Polarity and MODE with the the IEEE 1149.1 JTAG specification.

Figure 8: System contents for nios2_spi_pio_q16.

Figure 9: Configuration for altera_avalon_spi (spi_0) component of nios2_spi_pio_q16.

### hipe_spi

In order to use this module as a JTAG device the TAP controller was created (jtag_tap_ctrl, jtag_tap_fsm). The JTAG device in hipe_spi, Fig. 10, was created by combining the TAP controller, Fig. 11, with hipe_jtag, Fig. 14. Instead of requiring JTAG connections on the top of the module, the spi2jtag module, Fig. 15, was used to connect the JTAG device via SPI.

spi2jtag

spi_inf

jtag_tap_one_hot_state_inf

jtag_inf

hipe_jtag

jtag_tap_ctrl

Figure 10: Annotated RTL for hipe_spi.

Figure 11: RTL for jtag_tap_ctrl instantiated in hipe_spi.

Figure 12: JTAG Interface instantiated for tap in hipe_spi.

Figure 13: SPI Interface instantiated in hipe_spi.

### hipe_jtag

The jtag device that was chosen to demonstrate spi2jtag was from our Senior Design project, HIPE (Hardware Integrated Prototyping Environment). Understanding exactly how this module works is out of the scope of this document; however, a brief description will be provided. Basically the module provides two parallel vectors of ports, one in and one out, to the hardware designer; by default all signals on the input are driven at the output but by changing the state of the module the output can be changed. The state of the module can be changed by control signals from a JTAG TAP controller, see Fig. 11.

Figure 14: RTL for hipe_jtag instantiated in hipe_spi.

### spi2jtag

Provides two SPI slaves. One of these slaves should be selected to communicate with the TAP controller, and the other selected to communicate with the IR or DR. The TAP controller can be navigated by sending the TMS signal for each TCK pulse as the MOSI for that SCLK pulse. The second slave has MOSI connected to TDI and MISO to TDO (See Fig. 15).

The important portion of spi2jtag.sv is very terse in hdl and easy to understand by a knowledgeable hardware designer, and therefore is included below.

ee492\projects\spi2jtag\hdl\spi2jtag.sv 51: assign cs_n_tap = spi.cs_n[0]; // Shift out jtag_encoded_state 52: assign cs_n_jtag = spi.cs_n[1]; // Shift out Instruction Register or Data Register 53: assign cs_n_loopback = spi.cs_n[2]; // Loopback 54: 55: assign jtag.trst_L = 1'b1; // Unused Active low reset 56: assign jtag.tck = spi.sclk; // Test Data Clock 57: assign jtag.tdi = spi.mosi; // Test Data (Shift) in 58: 59: // Allow tdi to control Tap FSM when tap is enabled. 60: // When disabled the Tap will move to and stay at nearest stable state. (on tck) 61: // Will steady out to one of these states: RTI, SDR, PDR, SIR, PIR. 62: assign jtag.tms = ( (1'b0==cs_n_tap ) ? spi.mosi : // Test Mode Select 63: 1'b0 ); // Default 64: 65: assign spi.miso = ( (1'b0==cs_n_jtag ) ? jtag.tdo : // Test Data (Shift) out 66: (1'b0==cs_n_tap ) ? tapp_state_so_delayed : 67: (1'b0==cs_n_loopback) ? spi.mosi : 68: 1'b1 ); // Default

Figure 15: RTL for spi2jtag instantiated in hipe_spi.

### HDL Files

Each hardware module is listed with a brief summary of its purpose below.

- hipe_spi.sv

  - Instantiates hipe_rw and jtag_tap_ctrl as a complete JTAG Device.

  - Provides spi_inf which connects JTAG signals via spi2jtag.

- spi2jtag.sv

  - Provides two SPI slaves, the first controls the TAP controller via TMS and the second accesses the JTAG IR/DR registers via TDI/TDO, see Fig. 15.

- spi_inf.sv

  - A SPI interface with a configurable number of active low chip selects, see Fig. 13.

- jtag_inf.sv

  - A JTAG interface utilized by JTAG devices, see Fig. 12.

- shift_reg_async_ld.sv

  - A shift register with async load, which is utilized for shifting out current jtag_tap_state.

- jtag_tap_ctrl.sv

  - Provides control signals for TDR, controlled via jtag_inf,  and provides either internal IR or external DR between TDI/TDO based on TAP state, Fig. 11.

  - jtag_tap_fsm.sv

    - Implements JTAG finite state machine, Fig. 4.

  - jtag_tap_fsm_state_t.svh

    - 4-bit enum reg for JTAG states (IEEE 1149.1 JTAG), equivalent to JtagTap.h.

  - jtag_tap_fsm_one_hot_state_inf.sv

    - Provides one-hot state signals for JTAG TAP states  (IEEE 1149.1 JTAG), which can be used to control JTAG test data registers (TDRs).

- hipe_jtag.sv

  - Provide test data registers (hipe_rw.sv), which can be controlled by a TAP controller and accessed as a shift register, Fig. 14.

- dev_board_hiper.sv

  - instantiation of hipe_spi which connect various development board signals (GPIO, push buttons, switches, leds, and 7-Segment displays) through HIPE to allow any signals from an FPGA board to be controlled via software and provide a very visual demo. Not verified.

## HDL Components for Top:

Top files connect hdl modules with the development board (DE10_LITE).  Support was also provided for DE1-SOC, and the DE2, but all verification was performed on the DE10_LITE:

- top_hipe_spi.sv

  - Verified and used for final demo.

  - Instantiates nios2_spi_pio as SPI Master which interfaces with hipe_spi.

  - PIO of nios2_spi_pio  is connected in loopback through hipe_spi to verify functionality.

- top_spi2jtag.sv

  - Copy of top_hipe_spi before code cleanup. Outputs IR, tap state and SPI via GPIO. Verified.

- top_spi_master.sv

  - Instantiates nios2_spi_pio and connects SPI through GPIO.

  - Not verified; however, SPI transactions appeared to be correct on the OSCOPE.

- top_dev_board_gpio.sv

  - Instantiates dev_board_hiper.sv and connects this SPI slave via GPIO. Not verified.

- top_dev_board_nios2.sv

  - Instantiates dev_board_hiper as slave and nios2_spi_master  as master. Not verified.

- nios2_spi_master.sv

  - Minimal version of a SPI master via GPIO using nios2_spi_pio. Not verified.

- top_spi_slave.sv

  - Instantiates hipe_spi which intercepts Switches that drive LEDs by default; hipe_spi is controlled by a spi_inf ; the spi_inf was connected to GPIO.

  - Not verified; were not able to communicate with slave via GPIO from top_spi_master.

## Software Design Components:

Notably the software has been written as a library in (C++-03) to support jtag device interactions (ie. IR and DR shifts) using 8-bit SPI transactions through the Altera SPI core (3 wire serial) IP Component.  All SPI communication is performed through altera_spi_transfer() in altera_spi.h; to use the library with a different SPI Master/Driver, this function must be replaced.

### Spi2JtagApp

Provides an example usage of  the spi2jtag C++ Library with access to functionality via stdin/stdout. Allows the libraries core functionality to be used in a debug or demo scenario without having to recompile. By calling Spi2JtagApp::interactive() the user is able specify an action by entering a value 0x0-0xF, Fig. 16. There are four “registers” (a,b,c,d) that can be set and used as arguments for the various function calls. All input is accepted from the user as hex values via functions in scanf_x.h.

Figure 16: Functionality (left) available to user via terminal (right) connected to stdin/stdout.

### Software Files

Each software module is listed with a brief summary of its purpose below; paths are mentioned relative to the following directory from the repo: ee492/projects/spi2jtag/quartus/software/AlteraNios2/include.

- spi2jtag

  - Library for interfacing with spi2jtag module and hipe_spi.

  - spi2jtag.h

    - Combines all required includes for spi2jtag software components.

  - spi2jtag/Spi2JtagApp.h

    - Provides an interactive UI for the spi2jtag C++ Library via stdin/stdout.

  - spi2jtag/HipeJtag.h

    - Struct for accessing registers in hipe_jtag.sv via SPI.

  - spi2jtag/HipeRw.h

    - Intended to model hipe_rw.sv; however it was found easier to create HipeJtag.

  - spi2jtag/JtagDevice.h

    - Allows a ir_shift or dr_shift (see JtagTdr) to be performed with JTAG device via SPI.

  - spi2jtag/JtagTap.h

    - Models the JTAG TAP Controller and allows the state to be changed via SPI.

  - spi2jtag/PioTdr.h

    - Closed loop verification, expects pio_out to be connected through HIPE and back into pio_in which allows closed loop testing from software.

  - spi2jtag/spi_loopback_test.h

    - Sends SPI transactions with CS_LOOPBACK therefore MISO = MSOI in hardware.

- altera/ & bsp/

  - Files in these directories are specific to interfacing with Altera provided libraries and drivers. This concept increases the portability of the AlteraNios2 library created for this project.

  - altera/altera_stdint.h

    - Macros to convert altera datatypes (ie. alt_u8 ) to standard types (ie. uint8_t)

  - altera/altera_stdio.h

    - Macros isolate use of alt_stdio.h from altera, and make it easy to transition the code to use <stdio.h> instead.

  - altera/scanf_x.h

    - Mimics basic behavior of scanf for command line input using alt_getchar.

  - bsp/pio/altera_pio.h

    - Macros used to simply use of Quartus Qsys PIO component.

  - bsp/spi/altera_spi.h

    - Macros and functions used to simply use of Quartus Qsys SPI core component.

- jtag/JtagTapState.h

  - Enum for JTAG states (IEEE 1149.1 JTAG), equivalent to  jtag_tap_fsm_state_t.

- jtag/JtagTdr.h

  - Represents data register in Test Data Register, must be used with other modules to actually shift the data into the TDR.

- lib/

  - Directory contains code provided by other developers and used in this project.

  - lib/BinaryLiteral.h

    - Macro magic to convert binary string to its unsigned integer representation.

    - source: http://stackoverflow.com

  - lib/bithacks.h

    - Macros for performing basic binary operations on unsigned integers.

    - source: http://www.catonmat.net

- IF_DEBUG.h

  - Macro to wrap debug statements.

- pio2jtag/pio.h

  - Unused module where the altera_avalon_pio Qsys component was modeled.

- pio2jtag/pio2spi.h

  - Unused module which was intended to bit-bang SPI over PIO, but was not verified.

- UintUnion.h

  - Unused unions for representing an unsigned integer in different ways.

## Verification:

The hardware components for the JTAG TAP controller (jtag_tap_ctrl, jtag_tap_fsm) have been verified through both hardware and software based drivers and monitors.

### Verified jtag_tap_fsm via dev. board

Initially, a top file was created to isolate the jtag_tap_fsm design, switches were used to drive the Test Mode Select (TMS) and Test Reset (TRST_L); jtag_tap_fsm was driven by hand through all of the state transitions specified by the IEEE 1149.1 JTAG standard and the results were visually verified by observing the state on the HEX display.

### Verified TAP state transitions

The hardware modules (spi2jtag, jtag_tap_ctrl) were verified with JtagTap::test_transitions(); the success of this test also verified altera_spi_transfer() from altera_spi.h which was used for the SPI communication. The test performed SPI transactions with the spi2jtag module to transition through states, Fig. 17, in jtag_tap_fsm. These transactions were observed on the OSCOPE, Fig. 18, and the expected IR and tap state observed on the 7-segment hex displays.

Figure 17: Terminal output (left) from transitioning through a number of states (right).

Figure 18: Oscope capture of SPI transactions with JTAG TAP for transitions: {RTI->SIR, SIR->RTI}.

### Verified IR and DR shifts

The hardware module (hipe_spi) was verified with PioTdr.h; which expected PIO to be connected in loopback through hipe_spi. After writing a value to the PIO’s output via PioTdr::pio_out() the value on the PIO’s input can be read via PioTdr::pio_in() to determine the effect of hipe_spi. Specific IR and DR shifts were performed in PioTdr::tdr() and PioTdr::tdr_wen() to change the behavior of the hipe_spi instance and then the expected behavior was checked with PioTdr::check_pi(). Captures of specific SPI transactions show how the IR and DR value changed via SPI rather than JTAG, Fig. 19 and Fig. 20. The Instruction Register (IR) was also verified visually on the 7-segment hex displays.

Figure 19: Oscope capture of the following transactions: {tap=SIR, IR[4:0]=4’h2, IR[12:4]=0}.

Figure 20: Oscope capture of the following transactions: {tap=SDR, DR[8:0]=0, tap=RTI}

## Demo Recap:

A demonstration of the spi2jtag  embedded system in (Fig. 6) was performed by Nathan and Jordan on 2017-05-03, in the presence of Dr. Fourney.  This presentation was video recorded and will be producible upon request.  The block diagram presented at this demonstration is viewable in Fig. 21.

- Live demonstration

  - Showed messages from NIOS2 via JTAG UART that printed data from the IR and a TDR.

  - Development board displayed the values of a TDR which were send from NIOS2.

  - Development board displayed the state of the JTAG TAP FSM TDR which were changed from NIOS2.

- Data collected from OScope

  - The SPI signals for transactions of various JTAG TAP FSM transitions were observed on an O-SCOPE from GPIO pins on the FPGA dev board (Also described above).

  - The JTAG signals were observed on an on the FPGA dev board Hex displays.

## Future Work:

Use library with a different SPI driver by replacing altera_spi_transfer() in altera_spi.h.

jtag_tap_ctrl should provide ports to connect to the TDR’s TDO and TDI. The multiplexing performed in hipe_spi should be performed internally to this module.

The issue the causes an extra shift during SDR should be fixed in a different way. One possible solution could be for the JtagDevice to transition to CDR before shifting data rather than SDR. The current code essentially leaves the SDR or CDR state immediately when jtag_inf.tms goes low. The particular lines of interest are shown below:

ee492\projects\spi2jtag\hdl\jtag_tap\jtag_tap_ctrl.sv 37: assign ir_inf.sdr = (one_hot_state.sir & (~jtag.tms)); // Shift Instruction Register

ee492\projects\spi2jtag\hdl\spi2jtag\hipe_spi.sv 092: hipe_jtag #( 097: ) tdrs ( 102: .sdr({onehot_state.sdr & (~jtag.tms)}), 114: );

Currently JtagTap is limited to transitioning only to stable states. This limitation could be removed with a little bit of trivial work. The following functions and their uses should be considered.

ee492\projects\spi2jtag\quartus\software\AlteraNios2\include\spi2jtag\JtagTap.h 23: inline uint8_t jtag_tms(JtagTapStateEnum a, JtagTapStateEnum b){ 64: inline uint8_t jtag_transition(JtagTapStateEnum a, JtagTapStateEnum b){

## Appendix:

Figure 21: 2017-05-03-EE492-SPI2JTAG-DEMO-Figure-Engr-paper

Signals from bottom to top: MOSI, MISO, SCLK, CS_n[0] (TAP), CS_n[1] (DR/IR).

Signals from bottom to top: MOSI, MISO, SCLK, CS_n[0] (TAP), CS_n[1] (DR/IR).

Signals from bottom to top: MOSI, MISO, SCLK, CS_n[0] (TAP), CS_n[1] (DR/IR).